# Week 3 | Pre-Failure Signal Analysis

Windowed Feature Engineering, Leakage Control & Pre-Failure Signal Analysis

# Executive summary

The training-only matched analysis contains 518 positive/control pairs and evaluates 68 numeric fields. 38 fields meet the exploratory Benjamini–Hochberg FDR 0.05 threshold. Results are associations in synthetic data, not causal or externally validated estimates.

# Data and methods

Inputs: user-provided W03_signal_analysis.csv and W03_feature_stability.csv. The feature dictionary lists 67 engineered features; the signal analysis contains 68 numeric fields. The difference has not been reconciled with the underlying script.

Matching: script output reports 518 matched positive/control pairs using the training split. Exact matching criteria and whether controls are temporally independent require source-code verification.

For each feature, the provided results include positive/control means, mean paired difference, paired Cohen’s d, 95% confidence interval, p-value, BH-adjusted q-value, and an FDR significance indicator.

Leakage testing reported separately: 3,015 of 3,015 sampled historical recomputation checks passed across 45 robot-days. This does not establish the absence of all upstream timestamp, label, or split leakage.

# Leading observed signals

| Feature | Paired diff. | Cohen’s d | 95% CI (diff.) | BH q-value |
| --- | --- | --- | --- | --- |
| charge_cycles_30d_mean | +12.197 | +2.26 | [11.726, 12.647] | 1.11e-203 |
| charge_cycles_7d_last | +12.029 | +2.22 | [11.572, 12.456] | 6.58e-201 |
| charge_cycles_30d_last | +12.029 | +2.22 | [11.562, 12.503] | 6.58e-201 |
| unit_age_days | +12.784 | +2.21 | [12.237, 13.224] | 1.08e-199 |
| charge_cycles_7d_mean | +12.011 | +2.13 | [11.554, 12.423] | 2.38e-193 |
| battery_capacity_pct_7d_mean | -0.228 | -1.58 | [-0.241, -0.215] | 6.21e-142 |
| battery_capacity_pct_30d_mean | -0.206 | -1.56 | [-0.217, -0.195] | 1.66e-139 |
| wheel_wear_pct_7d_mean | +0.513 | +1.37 | [0.476, 0.545] | 8.44e-120 |
| wheel_wear_pct_30d_mean | +0.588 | +1.23 | [0.548, 0.630] | 7.09e-105 |
| motor_current_a_30d_mean | +0.051 | +0.92 | [0.046, 0.055] | 1.92e-70 |

Directionality: higher charge cycles, unit age, wheel wear and motor current, together with lower battery capacity, are associated with positive windows in the reported matched analysis. Closely related rolling windows and cumulative age variables should not be interpreted as independent mechanisms.

# Train-to-validation feature stability

The provided stability file reports train/validation coverage, means and standardized mean shifts for 68 numeric fields. Largest absolute shifts are shown below; shifts may reflect the chronological split and fleet aging rather than implementation errors.

| Feature | Train coverage | Validation coverage | Std. mean shift |
| --- | --- | --- | --- |
| charge_cycles_30d_mean | 99.7% | 100.0% | +2.497 |
| charge_cycles_7d_mean | 99.7% | 100.0% | +2.472 |
| charge_cycles_7d_last | 99.7% | 100.0% | +2.471 |
| charge_cycles_30d_last | 99.7% | 100.0% | +2.471 |
| battery_capacity_pct_30d_mean | 99.7% | 100.0% | -2.376 |
| battery_capacity_pct_7d_mean | 99.7% | 100.0% | -2.309 |
| battery_capacity_pct_7d_last | 99.7% | 100.0% | -2.185 |
| battery_capacity_pct_30d_last | 99.7% | 100.0% | -2.185 |

# Interpretation and limitations

Overlapping seven-day outcome windows create dependence among robot-days; nominal p-values and confidence intervals should be treated as exploratory, not confirmatory.

Age and cumulative charge-cycle features may reflect exposure duration or secular trends. Analyze short-horizon changes conditional on age and site before claiming early-warning value.

Pre-failure lead time is not quantified by these two CSVs. A lead-time curve requires event-aligned trajectories at several pre-fault horizons.

Only train-to-validation stability is supplied; cross-site stability and subgroup-specific effects are not established by these files.

The provided dictionary uses generic definitions for all 67 features. Exact computation formulas and source-time availability require review of W03_build_features.py.

All findings are derived from simulated fleet data and should not be presented as real-world robot reliability evidence.

# Next checks

Audit upstream event timestamps and seven-day label boundaries; inspect feature calculations and missingness by site; compare signals after conditioning on robot age; perform event-level or block-bootstrap uncertainty estimation; and quantify lead-time before Week 4 model development.

Sources: W03_signal_analysis.csv; W03_feature_stability.csv; W03_feature_dictionary.csv; terminal summaries provided for Week 3 pipeline and leakage tests.